ACCA QA Program

Policy Templates

October 2013

This guide and all earlier working/review drafts of this guide are protected by copyright.  By making this guide available for use and adoption by public authorities and others, ACCA does not waive any rights in copyright to this guide.  No part of this guide or earlier working/review drafts of this manual may be reproduced stored in a retrieval system or transmitted in any form by any technology without permission from ACCA.  Address requests to reproduce, store, or transmit to: Chris Hoelzel at the ACCA offices in Arlington, Virginia.

©2012, Air Conditioning Contractors of America

2800 Shirlington Road, Suite 300

Arlington, VA   22206

www.acca.org

ISBN

Disclaimer and Legal Notice

Diligence has been exercised in the production of this guide.  The content is based on an industry consensus of recognized good practices.  The commentary, discussion, and guidance provided by this publication do not constitute a warranty, guarantee, or endorsement of any concept, observation, recommendation, procedure, and process formula, data-set, product, or service.  ACCA and the document reviewers do not warrant or guarantee that the information contained in this publication is free of errors, omissions, misinterpretations, or that it will not be modified or invalidated by additional scrutiny, analysis, or investigation.  The entire risk associated with the use of the information provided by this guide is assumed by the user.

ACCA does not take any position with respect to the validity of any patent or copyrights rights asserted in connection with any items, process, procedures, or apparatus which are mentioned in or are the subject of this guide, and ACCA disclaims liability of the infringement of any patent resulting from the use of or reliance on this guide.  Users of this guide are expressly advised that determination of the validity of any such patent or copyrights, and the risk of infringement of such rights, is entirely their own responsibility.  Users of this guide should consult applicable federal, state, and local laws and regulations. ACCA does not, by the publication of this guide, intend to urge action that is not in compliance with applicable laws, and this document may not be construed as doing so.  Nothing in this manual should be construed as providing legal advice, and the content is not a substitute for obtaining legal counsel from the reader’s own lawyer in the appropriate jurisdiction or state.

Forward

ACCA recognizes the importance of documenting common business operations.  Clearly written company policies and procedures lay the groundwork for successful, consistent business operations. Consistent delivery of high quality HVAC installations is the primary purpose of the ACCA Quality Assured Program.

It is understood that some HVAC companies may not have “official” written company policies and procedures that address business prerequisites, business operations, training and certifications, and customer relations.  In response, the QA Steering Committee developed sample, customizable examples of policies and procedures for your consideration.  These policy templates are meant to be demonstrative of the types of policies and procedures that QA Participants are expected to have in place to meet minimum program requirements.  It is recognized that some companies may have policies in place at that are much more detailed than the one offered within this document.  The samples contained here are meant to be a resource, to be customized as deemed appropriate.

The way each company operates is unique, and it is understood that every owner, president, general manager, or company leader will desire to see things done their way.  Those personal preferences are what need to be documented regarding each of the QA Elements.  It is hoped that each company’s leadership will do more than just create a binder of policies that stays on a shelf collecting dust.  Rather, that your company policies, even those from very simple beginnings, will be implemented and used daily.  Then, as revisions are necessary due to growth or the adoption of good ideas, your living document will continue to define and describe “how we do things here”.

Introduction

The QA Policy Templates contain samples for the written policies and procedures that the QA Participant’s company agrees to consistently follow in order to maintain compliance with the ACCA QA Elements.  The policy templates are examples of the minimum requirements, and it is understood that that there may be policies in place at some companies that are much more detailed and much more complete than the ones offered within this document.   If there is a conflict between any elements of the QA Policy Template and any federal, state and/or local rule or regulation, the controlling rule or regulation supersedes the QA Policy Template.  Additionally, some QA Elements are not required in all jurisdictions, thus these policies are “As Applicable”.

This document contains each of the QA Elements detailed in the ANSI/ACCA 5 QI – 2010 (HVAC Quality Installation Specification) Standard.  Following the cited QA Element is a list of the primary elements, and a sample policy.  Each sample policy contains wording that is bolded and underlined, these passages describe fictitious job descriptions, locations, or other elements that vary from company to company.  Each QA Participant may modify their policy to meet the company’s unique, regional, or other operational requirements.

As you read through the document you may note:

- We don’t have that job description, or

- That’s not how we perform that task, or

- We don’t have a location in our office like that.

In those instances, modify this document to reflect how things operate in your company.  Additionally, if the example policies spark a thought, concept, or idea that should be implemented, use it as is deemed best.

Appendix A is a compiled policy manual for the mythical Bob’s Heating and Air.  It is hoped that this document will be easier for you to cut and paste as you begin your editing process to reflect the actual job descriptions, locations, and procedures in your company.

It is recommended that after completing the company’s policies, that your legal counsel review same to ensure compliance with regional or state law.

Drafting your company’s policy manual is only the first step in complying with the QA Program Requirements. The next three steps are repeated continuously: train company employees to comply with the policy, verify that they are complying, and take necessary corrective action if employees fail to follow a policy.  Corrective action may be disciplinary in nature however; it may be that there is a better way of doing things.  It is hoped that employees that are in noncompliance with your company policies will be able to offer improvements.  Thus step three would be to revise the company policy, and begin the three step process again: train, verify, correct.  The information contained in this document provides a minimum baseline for the written policies and procedures, but there is nothing “magical” in the words contained in these sample policies.  These printed words will not transform a company into a well-organized machine.  However, as compliance to company policies is enforced, constancy will be the result.  Following good policies will yield consistent quality that customers will appreciate and recommend to others.

TABLE OF CONTENTS

FORWARD	i

INTRODUCTION	ii

QA BUSINESS TEMPLATES

QA-1 Business Prerequisites

QA-1.1 Licensing or Registration	1

QA-1.2 Insurance	3

QA-1.3 Code Requirements	5

QA-1.4 Refrigerant Certifications, Training, And Equipment	7

QA-1.5 Hazardous Materials Regulations	9

QA-2 Business Operations

QA-2.1 Employment	11

QA-2.2 Safety Programs	13

QA-2.3 Fleet Management	15

QA-2.4 Quality Installations	17

QA-2.5 Quality Maintenance	19

QA-2.6 Quality Restoration	21

QA-2.7 Instrumentation/Measurement Tools	23

QA-3 Training and Certification

QA-3.1 Continuing Education	25

QA-3.2 Certification	27

QA-4 Customer Relations

QA-4.1 Interactions With Building Owners And Homeowners	29

QA-4.2 Warranties	31

QA-4.3 Service Agreements	33

APPENDICES

Appendix A:  Bob’s Heating and Air Compiled Sample Policy	35

| QA-1.1 LICENSING OR REGISTRATION [ACCA 5 QI – 2010] The contractor shall possess all required statutory/regulatory licenses and other state or local registrations for the business as dictated by the requirements in each jurisdiction where the contractor does business. All such items are to be current. |  |
| --- | --- |
| Primary Elements As applicable State, Regional, City Mechanical License (including sheet metal, gas fitters, etc.). State, County, City, Municipality Business Registrations or Licenses. Where are the licenses and registrations stored/files/posted? Who is charged with keeping them current? |  |
| Sample Policy #1 | Bob’s Heating & Air located at 123 Main Street, Comfort City is required by the state of XXXX to have a State Mechanical License and a State Business License. Additionally, YYYY County requires the company to have a County Business License. Our licenses are located on the wall, behind the reception desk, at the entrance of our office suite. The VP of Operations is responsible for renewing the State Mechanical License every three years. It is the duty of the President of Bob’s Heating & Air to renew the state business registration annually. |
| Sample Policy #2 | XXXXX does not require licensing to install HVAC systems (or systems that do not exceed $15,000), thus Bob’s Heating and Air does not maintain a license. YYYY Municipality does not require a business registration, thus Bob’s Heating & Air does not maintain a business registration. |

| QA-1.2 INSURANCE [ACCA 5 QI – 2010] The contractor shall carry at least the statutory levels of insurance and applicable bonds required by the appropriate jurisdiction where the company offers its services, as well as all applicable contractually required insurance. Such insurance coverage shall include, but is not limited to: - Liability, - Worker’s compensation, - Company vehicles, and - Bonds. |  |  |
| --- | --- | --- |
| Primary Elements General liability insurance. Automotive insurance. Worker’s compensation insurance as required. Performance bond as required. Location of insurance policies and performance bonds stored/maintained. Identify who is responsible for renewing the insurance policies and bonds. |  |  |
| Sample Policy | Bob’s Heating & Air is required to have general liability, automotive, and worker’s compensation insurance policies by the state of XXXX. Performance bonds, which are required for projects in excess of $1,500,000 or if stated in a particular project contract, are purchased. Insurance policies are filed in the ‘Insurance’ folder located in cabinet 1A in the files room in the back of the office suite. A copy of each contract that has bonding requirements is filed in the ‘Bonding Required’ folder located in cabinet 1A, in addition to being in the regular Project File kept in the same files room. It is the duty of the President of Bob’s Heating & Air to renew the policies annually. |  |
| Sample Policy – Optional Amendment I |  | Bob’s Heating & Air is exempt from the worker’s compensation insurance in the state XXXX due to its status as sole proprietor, single owner, no employees and is not required to file an exemption with the state. |
| Sample Policy – Optional Amendment II |  | Bob’s Heating & Air is exempt from the worker’s compensation insurance in the state XXXX due to its status as a sole proprietor, single owner, no employees. Bob’s is required to file an exemption with the state, a copy of the approved exemption is maintained in the Bob’s office. |

| QA-1.3 CODE REQUIREMENTS [ACCA 5 QI – 2010] All work performed by the contractor shall comply with all required state and local building codes, energy codes, and related regulations in each jurisdiction where the contractor does business, including, but not limited to: - HVAC equipment sizing - HVAC selection - HVAC installation - HVAC servicing, and - HVAC maintenance. |  |
| --- | --- |
| Primary Elements Codes followed: - State - Local amendments Available copies of the applicable: - Code - Amendments Location where the code is kept. Responsible person who maintains codes. Authorities having jurisdiction that are currently served. |  |
| Sample Policy #1 | Bob’s Heating & Air complies with the codes cited by the authority having jurisdiction (AHJ), XXXX (state) and YYYY County. The cited codes are the 2012 IRC and the 2012 UMC, including the county amendments. For work performed in other states or AHJs, Bob’s will coordinate with that AHJ to ensure copies of the applicable code are on hand, understood, and available. Copies of the code are kept in our reference library in the staff conference room. Compliance achieved by: - Educating team leaders & conducting inspections of random samples of their jobs (see QA-2.4 Quality Installation) - Maintaining a close working relationship with the building department - Using examples of ‘red tag’ or other noted instances of failure to comply in weekly team leader meetings - The Operations Officer coordinates annually with local code official (per instance for our-of-area prospects) to learn of any new or unique code changes / amendments |
| Sample Policy #2 | The jurisdiction having authority in which Bob’s Heating & Air operates has not adopted a building code and does not have a code enforcement agency. Therefore Bob’s has determined to follow the building code adopted by the state in which we do business. The cited codes are the 2009 IRC and the 2012 UMC, including the amendments. For work performed in other states or AHJs, Bob’s will coordinate with that AHJ to ensure copies of the applicable code are on hand, understood, and followed. |

| QA-1.4 REFRIGERANT CERTIFICATIONS, TRAINING, AND EQUIPMENT [ACCA 5 QI – 2010] All technicians, or third-party subcontractors, who handle refrigerants, are to recover/recycle refrigerants in compliance with EPA regulations (i.e. Section 608). The contractor is responsible for: - Use of operational Recover/Recycle equipment, - Use of operational evacuation equipment capable of maintaining a vacuum of at least 500 microns, - Ensuring company technicians who recover refrigerants, recycle refrigerants, or charge refrigerants to HVAC systems possess all required EPA refrigerant certifications (i.e., I, II, III or Universal), - Providing required recover/recycle equipment to company technicians who handle refrigerants, - Ensuring that pertinent technicians are trained in the correct use of recover/recycle equipment, - Ensuring that pertinent technicians are trained in the correct use of evacuation equipment, - Ensuring that company refrigerant records are kept in compliance with Section 608 EPA regulations. |  |
| --- | --- |
| Primary Elements Evidence of compliance with EPA/Section 608. Verification process for technician training. Identify who provides evacuation and recover/recycle equipment. Identify where refrigerant handling records kept. Identify who organizes and maintains refrigerant handling records? |  |
| Sample Policy I | Bob’s Heating & Air provides all recover/recycle and evacuation equipment to technicians working with refrigerants. In order to work with refrigerants, all employees must possess the applicable EPA Section 608 card. The Service Manager ensures a copy of the card is maintained cards in the employee’s company file, maintained in the Dispatcher’s office. All new technicians must demonstrate to the Service Manager the ability to correctly: - Use recover/recycle and evacuation equipment, - Store and transport refrigerants, - Use a micron gauge (technician supplied), - Change the vacuum pump oil (disposal in compliance with QA-1.5 Hazardous Waste Policy), and - Complete company records of refrigerant handling: - Type (R-22, R-410A, etc.) - Volume, by weight - Date - Location (and zone if applicable) A mater record of incidental refrigerant use is maintained by each technician. These records are submitted to the Dispatcher. All refrigerant use records are maintained in the Dispatcher’s office. |
| Sample Policy II | Bob’s Heating & Air requires technicians to provide all recover/recycle and evacuation equipment to technicians working with refrigerants. In order to work with refrigerants, a technician must possess the applicable EPA Section 608 card. All new technicians must demonstrate to the Operations Manager the ability to correctly: - Use recover/recycle and evacuation equipment, - Store and transport refrigerants, - Use a micron gauge (technician supplied), - Change the vacuum pump oil (disposal in compliance with QA-1.5 Hazardous Waste Policy), and - Complete company records of refrigerant handling: - Type (R-22, R-410A, etc.) - Volume, by weight - Date - Location (and zone if applicable) A mater record of incidental refrigerant use is maintained by each technician. These records are submitted to the Dispatcher. All refrigerant use records are maintained in the Dispatcher’s office. |

QA-1.5 HAZARDOUS MATERIALS REGULATIONS [ACCA 5 QI – 2010] The contractor shall adhere to all applicable federal, state and local pollution codes and regulations related to handling hazardous materials, and will develop and comply with guidelines and policies for the safe and proper handling of hazardous materials and substances at the company workplace(s) and work sites, including: - Proper storage of hazardous materials and substances at the workplace, - Proper transportation of hazardous materials and substances to and from different sites, - Proper use of hazardous materials and substances by the company, - Proper disposal of hazardous materials and substances by the company, - Following proper procedures for treating and cleaning up discharges and accidental spills, - Ensuring material safety data sheets (MSDS) are used, and - Ensuring personnel use appropriate safety equipment.

Primary Elements Disclosure of hazardous materials handled (oil, mercury, refrigerants, PCPs, solvents...) Company policy addressing: - Hazardous materials: - Storage, - Transportation, - Use, - Disposal - Treating and cleaning up discharges and accidental spills Material Safety Data Sheets used. Proper safety equipment available.

| Sample Policy | Bob’s Heating & Air commonly works with hazardous materials (HazMats) such as lubricants, mercury, PCBs, solvent, and refrigerants. In order to safely work with these materials, Bob’s distributes a binder of Material Safety Data Sheets (MSDS) for all hazardous materials with which we work. We obtain the MSDS via the following methods: - Requesting them directly from the manufacturer at the time of the purchase - Online MSDS databases such as www.msdsxchange.com MSDS have the following information clearly addressed on them: substance/material identification and manufacturer business info; composition and ingredients; hazards identification; first aid measures; firefighting measures; accidental release measures (spills); handling and storage; exposure controls/personal protection; physical and chemical properties; stability and reactivity; toxicological information; ecological information; disposal considerations; transport information; regulatory information; other information. Though not all MSDSs contain all of this information, the majority of MSDS for the materials we work with do, to a significant degree. Our Chief Safety Officer reviews the MSDS and provides supplemental guidance if necessary. MSDS are kept in large, clearly labeled binders on each work vehicle. This allows easy access to essential information for all employees that will need to work with hazardous materials. They are organized by function (lubricant, refrigerant, PCB, mercury…), then manufacturer, then product identification. |
| --- | --- |

| Sample Policy (continued) | Our policies on storage, transportation, use, and disposal of hazardous materials, as well as treating and cleaning up discharges and accidental spills are based on the applicable MSDS sections. As applicable, spills require the area to be cleared of non-essential employees until the spill is taken care of as per MSDS, manufacturer’s instructions, and/or regulatory requirements. . Transportation of hazardous materials also follows the MSDS recommended method as well as U.S. DOT requirements. An internal safety training program led by the Chief Safety Officer is conducted in conjunction with the continuing education program on a quarterly basis, with a 1.5 hour module at the time of hiring. This safety training is intended to (1) keep all technicians current on the proper safety procedures when handling hazardous materials that they are likely to encounter, (2) to ensure that they can readily understand the information contained on MSDS, and (3) are using the appropriate safety equipment when it is necessary. Our disposal procedures for hazardous materials are the following: - Capacitors are disposed of at the county hazardous materials yard - Install Teams and Technicians place capacitors with PCBs in a green bucket by the Dispatcher’s window. - The Warehouse Supervisor takes them to the county hazardous materials yard. Or, - The Install Team Leader keeps them in a marked bin on the truck and then disposes of them when they dump trash from the job site at the junk yard next door. - Mercury (thermostats) are disposed of at a local wholesale distributor - Install Teams and Technicians place thermostats with mercury in an orange bucket by the loading bay door. - The Installation Manager takes them to the county hazardous materials yard. Or, - The Install Team Leader keeps them in a marked bin on the truck and then disposes of them when they pick up materials for the next job site. - Vacuum pump oil is disposed of at the local “Oil Change” shop. - Install Teams and Technicians drain their vacuum pumps into an old vacuum pump oil container painted blue. Every morning the Helper empties it into the black oil bucket by the Warehouse Supervisor’s window. - The Warehouse Supervisor takes the bucket to the Oil Change shop every other Thursday or when the bucket is full. Or, - The Install Team Leader empties the blue oil container at the Oil Change shop every Friday. SAMPLE 1: Recovered Refrigerants - Install Teams and Technicians place recovered refrigerant in DOT approved containers provided by Bob’s Heating and Air. - On the tag attached to the container, the Install Team Chief or Technician must document each instance that refrigerant is recovered, noting the location and volume of refrigerant recovered. |
| --- | --- |
| Sample Policy (continued) | - They take the refrigerant cylinders to the local HVAC distributor’s where they are exchanged for empty containers (they must remove the tag from the full cylinder), - The Install Team Chief or Technician must turn in the tag in order to receive a new cylinder of virgin refrigerant. SAMPLE 2: - Install Teams and Technicians place recovered refrigerant in DOT approved containers provided by Bob’s Heating and Air. - On the refrigerant log attached to the container, the Install Team Chief or Technician must document each instance that refrigerant is recovered, noting the location and volume of refrigerant recovered. - The Parts Runner will collect will collect the full cylinder and the refrigerant log from the Team Chief or Technician when they deliver cylinders of new refrigerant. - The Parts Runner will provide the refrigerant log and full cylinder to the Warehouse Supervisor. - The Warehouse Supervisor will recover the refrigerant into the 500lb shop cylinder. - The Warehouse Supervisor will call “Refrigerant for Cash” recycling service to pick up the full cylinder and drop off an empty. - The Warehouse Supervisor will maintain the logs for all refrigerants recovered and issued. Personal protective equipment required by Bob’s Heating and Air is listed in the Safety Programs Policy. |

| QA-2.1 EMPLOYMENT [ACCA 5 QI – 2010] The contractor shall perform reference and criminal background checks and ensure that all employees meet appropriate employment eligibility requirements. |  |
| --- | --- |
| Primary Elements Reference check. Criminal background check. Employment eligibility check. Identify who performs background and eligibility checks. |  |
| Sample Policy | Bob’s Heating & Air complies with federal employment eligibility requirements by completing and retaining Form I-9 and a copy of the employee’s identification in their employee file located in the Office Manager’s office. Bob’s conducts criminal background checks using XXXX Agency, and reference check before offering employment to an applicant. During reference checks, the Operations Manager calls the reference to: - The relationship listed with the applicant is correct, - Confirm the applicants dates of employment, if applicable, and - Request any information that may be useful when considering the employment of the applicant. As applicable, employees are assigned to duties that ensure compliance with Megan’s Law. |

| QA-2.2 SAFETY PROGRAMS [ACCA 5 QI – 2010] The contractor shall ensure employees receive the appropriate type and level of safety training, including, but not limited to: - Fleet safety, and - Injury prevention. |  |  |
| --- | --- | --- |
| Primary Elements Regarding fleet safety and injury prevention training (OSHA), identify: - Instructor - Subject matter presented - Attendance Location of safety records. Identify who organizes and maintains the records. |  |  |
| Sample Policy | Bob’s Heating & Air is required by the state of XXXX to have a safety program that meets the training, records keeping, and other requirements set by OSHA and State OSHA. All records related to Bob’s Safety Program are maintained by the VP. Human Resources in their office. The VP of Operations is responsible for all Safety Program training related to fleet safety (maintaining safe driving distance, seat belt use, use of mirrors, lane changing, etc.) and personal injury prevention (ladder safety, head/hand/foot protection, soldering safety, medical/first aid, etc.). All records related to safety training are maintained in the V.P Human Resources office. Bob’s requires the following personal protection equipment (PPE): - Safety glasses - Safety harness, in circumstances required by OSHA - Plastic/latex gloves (sealants) - Hearing protection - Hard hats, in circumstances required by OSHA - Leather gloves (sheet metal) - Steel toed shoes Bob’s provides the safety glasses, safety harness, plastic gloves, and hearing protection. Employees are required to provide their hard hat, leather gloves, and safety shoes. These items are expected to be in good condition or to be replaced by the employee at the discretion of Field Supervisor. The list of PPE required by each employee is reviewed at the time of hiring. A signed copy of this record is kept in the Safety Program’s records (V.P Human Resources office). |  |

| QA-2.3 FLEET MANAGEMENT [ACCA 5 QI – 2010] The contractor shall ensure dedicated company vehicles meet federal, state, and local laws and regulations and are properly maintained. Such compliance shall include, but not be limited to: - Ensuring that company vehicles are marked in accordance with legal requirements, and - Ensuring company vehicles are properly maintained in accordance with legal requirements. |  |
| --- | --- |
| Primary Elements Federal, state, local regulations for vehicle maintenance. Vehicles properly registered. Vehicles properly marked. Vehicle inspection records. Location of fleet management records. Identify who organizes and maintains the records. |  |
| Sample Policy | Bob’s Heating & Air complies with XXXX state requirements for our fleet to undergo an annual emissions test for each vehicle. Additionally, all company vehicles are registered with XXXX Department of Transportation, and all employees who drive these vehicles are required to have a Class C license. Our company employs Ace Automotive for fleet maintenance and keeps copies of all maintenance records in our records room (in addition to the records kept by Ace). The records are kept current by our Operations Officer and are kept in the filing cabinet designated Fleet Documentation. As per XXXX state requirements, all company vehicles are marked as required in Federal Regulation 390.21 (company name and US DOT number). |

QA-2.4 QUALITY INSTALLATION [ACCA 5 QI – 2010] The contactor shall ensure that HVAC equipment replacements and new equipment installations comply with ANSI/ACCA Standard 5 (HVAC Quality Installation Specification), and shall specifically develop and comply with policies which ensure that the contractor is following all Quality Installation (QI) Specification requirements: - Ventilation requirements, - Heat-gain and heat-loss load calculations, - Properly-sized HVAC equipment, - Ground heat exchangers, - Properly matched systems, - Airflow through the indoor heat exchanger, - Water flow through the indoor heat exchanger, - Refrigerant charge, - Electrical requirements, - Combustion equipment is “on-rate”, - Venting of combustion gases, - System operational and safety controls, - Air ducts are sealed (CFM), - Room airflow, - Water flow, - Customer documentation, - Owner and/or operator education.

Primary Elements Documentation required for installation files (HVAC system design files, company start-up sheets). Procedures for compliance with ACCA 5 QI requirements. Location of installation files records. Identify who organizes, reviews, and maintains the records.

| Sample Policy #1 (HVAC installations in new or existing homes) | Bob’s Heating & Air ensures that every HVAC system installation for ENERGY STAR intended homes is compliant with ACCA 5 QI Standard. Similarly, Bob’s will always propose following the ACCA 5 QI Standard when replacing an existing system in a residence, and will comply with the standard’s requirements when that option is selected. System designers and installation technicians are thoroughly trained and comfortable with the requirements of the Standard. HVAC System Design The system designer creates an installation file that includes the load calculation, expanded manufacturers equipment performance data, duct sketches, duct sizing calculations, and the AHRI certificate for the system to be installed1. The file is then given to the Operations Manager. Bob’s uses the following procedures to design HVAC systems: - Ventilation requirements are calculated using ASHRAE 62.2 - Heating and cooling loads are calculated using ACCA Manual J8 - Equipment is selected using the procedures in ACCA Manual S and the information from the OEM expanded performance data - Ground heat exchangers are designed in consideration of the elements in ACCA 5 QI §3.4.1.ii - An AHRI certificate is printed and filed for all systems |
| --- | --- |

| Sample Policy #1 (Continued) (HVAC installations in new and existing homes) | HVAC System Installation During the HVAC system installation, measurements are recorded on the company start-up checklist (for ENERGY STAR homes, the current ENERGY STAR checklist is used) in accordance with the procedures defined in the Technician’s Guide for Quality Installations2. Homes intended for ENERGY STAR qualification have well-marked, pre-drilled holes where static pressure measurements are taken. For HVAC system replacements, the lead technician is responsible for demonstrating how to operate the thermostat, how to change the filter, and for leaving the Bob’s Heating and Air Customer Information Sheet that explains the warranty, maintenance program, and the company contact information. Upon completion of a system installation, the Start-up Technician submits the company startup checklists (and other supporting documentation as needed) to the Operations Manager. Ensuring HVAC System Installation Compliance The Operations Manager visits every job upon completion to ensure compliance with this policy. Projects lasting more than three (3) days receive at least one jobsite visit during the visit. As a minimum, projects lasting longer than five (5) days receive an onsite job review at least on every third day. OR…. The Operations Manager visits every job upon completion to ensure compliance with this policy. OR…. The Operations Manager randomly selects 30% of all jobs to visit (upon completion) to ensure compliance with this policy. Ensuring Compliance The Operations Manager follows the verification process found in the ACCA Standard 9 Quality Installation Verification Protocols (ACCA 9 QIvp) to assure installation compliance. The Operations Manager observes the protocols for the ‘Administrator/Verifier’ in ACCA 9 QIvp. Their duties are: - Inspect every installation file for completeness and compliance with QI Standard - Random in-field verification of system installations. The Operations Manager follows the evaluation protocols of Section 5.0 of the ACCA 9 QIVP Standard and uses the tables/checklist in that section for assistance in determining compliance. Upon verification completion the Operation’s Manager is responsible for storing the installation file in the customer’s file located in the Dispatcher’s office. |
| --- | --- |
| Sample Policy #2 (HVAC in Existing Homes) | Bob’s Heating & Air ensures that every client is offered an HVAC system installation compliant with ACCA 5 QI. Clients that decline proper system installation and desire to accept a non-QI equipment replacement must sign a disclosure statement and acknowledge the disadvantages of improperly installing the HVAC system (A copy of the disclosure statement can be found Policy QA-4.1 Interactions With Building Owners And Homeowners on page 18). HVAC System Design/ Installations/ Oversight See Sample Policy #1 |

| Sample Policy #3 (Sub-contracted HVAC system installations) | Sub-Contracting HVAC System Design System design is occasionally sub-contracted to Harry’s Mechanical Designs or (as contracted) Bob’s Heating and Air will install to the design provided by others. - Based on seasonal workload, Bob’s intermittently contracts with Harry’s Mechanical Designs to calculate heating and cooling loads, select equipment, design and size duct systems, etc. Bob’s Heating and Air acknowledges responsibility for the HVAC system designs performed by Harry’s Mechanical Design. Bob’s maintains full responsibility for each installation’s designs (loads, equipment selection, duct design, etc.). - Designs provided by others: Some Homebuilders have the HVAC system designed by others. Additionally, some homebuilders make bulk purchases of materials and equipment, addressing equipment selection. Bob’s Heating and Air merely provides labor for the installation of the HVAC equipment per the design supplied by the Homebuilder. In those instances, Bob’s Heating and Air ensures that contractual stipulations place responsibility for HVAC system design (including equipment selection as applicable) with the Homebuilder. HVAC System Installations Bob’s Heating & Air occasionally sub-contracts portions of HVAC system installation to various companies. Bob’s Heating and Air acknowledges responsibility for those portions of the sub-contracted HVAC system installation. Bob’s accepts full responsibility for each installation and ensures compliance to the QI Standard as mentioned in Policy #1 above. |
| --- | --- |

| QA-2.5 QUALITY MAINTENANCE [ACCA 5 QI – 2010] The contractor shall develop and comply with policies and procedures ensuring that maintenance work performed is in accordance with ANSI/ACCA Standard 4 (Maintenance of Residential HVAC Systems) or ANSI/ASHRAE/ACCA Standard 180 (Standard Practices for Inspection and Maintenance of Commercial Building Systems), and with OEM recommendations: - Inspections of the HVAC system, - Repairs and corrective actions, and - Inspection, repairs, cleanings, and other maintenance tasks. |  |  |
| --- | --- | --- |
| Primary Elements Policy that complies with ACCA 4 QM or ANSI/ASHRAE/ACCA Standard 180. Documentation. Company worksheets. Location of installation files records. Identify who organizes and maintains the records. |  |  |
| Sample Policy | Bob’s Heating & Air only performs residential work, so we follow ACCA 4 QM for maintenance of HVAC systems. Bob’s uses a series of checklist that can be followed by our technicians in the field to conduct comprehensive system maintenance. The technician conducts a thorough inspection of the HVAC equipment per the checklists in the standard. Upon completion of the inspection, the technician will inform the owner/building operator of any deficiencies (code violations as well as the performance objectives for the equipment based on make, model, start-up date, etc.). The technician will then perform the corrective actions approved by the customer (the customer is the person paying for the approved work). Bob’s maintenance checklists document the items cited in §5.1.1 to §5.1.7 of ACCA 4 QM (the home’s equipment inventory, equipment inspection checklist, code violations, performance objectives, relevant external conditions, and inaccessible items). Corrective actions are based on the procedures from OEM. Every employee that services and maintains HVAC equipment is given a copy of ACCA 4 QM. QM documentation is kept in the service record of each client, and the service records are maintained by the Dispatcher in the Maintenance/Service file cabinet in the Dispatcher’s office. |  |

| QA-2.6 QUALITY RESTORATION [ACCA 5 QI – 2010] The contractor shall develop and comply with written policies for the relevant work undertaken regarding duct cleaning and other extraordinary system restoration work in accordance with ANSI/ACCA Standard 6 (Restoring the Cleanliness of HVAC Systems). |  |
| --- | --- |
| Primary Elements Policy in accordance with ACCA Standard 6 - Establishing criteria for HVAC system cleaning - HVAC system restoration protocols - Post cleaning verifications - Post cleaning products and usage Location of installation files records. Identify who organizes and maintains the records. |  |
| Sample Policy | Bob’s Heating & Air follows strict steps when restoring and HVAC system that are in accordance with ACCA Standard 6. They are the following: - A trained technician will conduct an initial survey to assess the practicality of a restoration, and to see determine possible problems. The survey includes: - Visual system cleanliness evaluation (interior and exterior), - Inspection of drains, and - Inspection of room where the system is housed. - System performance shall be measured in order to compare the pre- and post-restoration performance. If there is a possible impact to the customer’s health, an indoor air quality expert will be asked to provide a formal report. - Bob’s Heating & Air follows the protocols for system restoration that are found in Section 5.0 (System Restoration Protocol) of the ACCA 6 QR which covers the following topics: - Worker protection, - General principles (containment, physical dislodgment, particle transport, contaminant removal), - Cleaning equipment (particulate collectors, pressurized air supply, HEPA filtered canister vacuums, duct surface brushing systems, power washers), - Cleaning processes and procedures (continuous pressure differential, - Particulate collectors, - Fibrous glass duct insulation cleaning method, - Uniformity in cleaning, - Fine particulate extraction, - Electric heating and electric reheat coils, - Removal of contaminated materials from HVAC systems, - Building isolation, - Work sequence, - Protecting cleaned surfaces, - Disposal of existing HVAC filters, - Cleaning of HVAC system components and subsystems (cooling/heating coils, fan blades and fan housing, motors, air handling units and interior heat exchanger cabinet surfaces, drain pans and drain line, HVAC airflow control components, registers, diffusers, and grilles, ductwork systems, rigid ducts, |

| Sample Policy (Continued) | Fiberglass lined or duct board ductwork, flexible non-metallic ducts), and duct service openings and closures (metal ducts, service opening and closure fiberglass duct board ducts, flexible ducts). - The Team Leader verifies the cleanliness of the system by turning it on, measuring performance, recording measurements, and comparing it to pre-cleaning performance. If an expert was called in during the initial system evaluation, an independent third party reviewer will be asked to review the system for health reasons. - The Operations Manager visits every job upon completion to ensure compliance with this policy. Projects lasting more than three (3) days receive at least one jobsite visit during the visit. As a minimum, projects lasting longer than five (5) days receive an onsite job review at least on every third day. OR…. The Operations Manager visits every job upon completion to ensure compliance with this policy. OR…. The Operations Manager randomly selects 30% of all jobs to visit (upon completion) to ensure compliance with this policy. - Upon completion, the owner (or their agent) visually inspects the system cleanliness. - Bob’s Heating & Air follows the exact directions for post cleaning products and usage that are found in Section 7.0 (Post Cleaning Products and Usage) of Standard 6 which covers the following topics: resurfacing products, closure products (mastics, tapes for rigid fiberglass duct board ducts and other openings), sanitizers, disinfectants and other antimicrobial products. Every employee that will be doing restoration work has a personal copy of Standard 6 issued to them. Restoration records are kept in the service record of each client, and the service records are maintained by the Dispatcher in the Maintenance/Service file cabinet in the Dispatcher’s office. |
| --- | --- |

| QA-2.7 INSTRUMENTATION/MEASUREMENT TOOLS [ACCA 5 QI – 2010] The contractor shall develop and comply with policies requiring that appropriate instrumentation/measurement tools are available for each service offered by the contractor. Further, the contractor shall maintain calibration and service records for instruments and test equipment that require calibration and maintenance. |  |
| --- | --- |
| Primary Elements List of appropriate instrumentation/measurement tools to complete services offered. Proper instrument calibration and service records. Location of installation files records. Identify who organizes and maintains the records. |  |
| Sample Policy | Bob’s Heating & Air provides each technician with a digital anemometer, a flow grid, a digital temperature gauge, and a multimeter (voltmeter and ammeter) in order to properly install, maintain, restore, and test HVACR systems in the field. Digital meters are calibrated according to the manufacturer’s recommendation. We have one blower door that is kept in the office and can be signed out by technicians prior to using it. The Operations Manager keeps the calibration and service records for company owned instruments and equipment in the calibration file located in his/her office. Each Technician is in charge of keeping their instruments calibrated (if needed) and serviced per equipment manufacturer’s guidelines. The Operations Manager also spot-checks the technicians’ personal tools to ensure that they are properly calibrated and serviced. |

| QA-3.1 CONTINUING EDUCATION [ACCA 5 QI – 2010] The contractor shall ensure that company personnel receive: - A minimum of 6 equivalent hours of application and technical training per year for each company salesperson/designer, and - A minimum of 12 equivalent hours of technical training per year; or technician maintains certification by an HVAC industry-recognized and accepted certification program (e.g., NATE). |  |
| --- | --- |
| Primary Elements Regarding instruction provided in-house, identify: - Instructor - Subject matter presented - Attendance Regarding instruction provided by others, provide a certificate or other documentation to demonstrate: - Instructor - Subject matter presented - Attendance Location of training records. Identify who organizes and maintains the records. |  |
| Sample Policy | Bob’s Heating & Air provides a comprehensive continuing education program delivered by the Continuing Education Officer. Our salespeople and designers take a three hour course every six (6) months. Our technicians are required to take a three (3) hour technical education course every quarter. A small comprehension assessment is given by the instructor at the end of each course. In order for a technician to opt out of the continuing education course, they must maintain their current NATE certification. They must provide us with their NATE number (to be included in the personnel file) for corroboration, or we verify each technician’s Continuing Education Units annually. If a technician misses a quarterly training class they must make up the continuing education hours by attending a training class provided by an OEM, an HVAC distributor, a tech school, or ACCA and submitting corroboration of attendance. These training classes cannot replace more than three (3) hours of our company’s in-house continuing education courses to fulfill the six (6) and twelve (12) hour annual requirements. Records of the courses (class material, subjects covered, date, instructor, attendees) are kept in the records room of our office suite in the continuing education cabinet. These records shall be maintained for at least five (5) years. The calendar of future courses is kept in the staff break room. |

| QA-3.2 CERTIFICATION [ACCA 5 QI – 2010] The contractor shall ensure employees maintain proper certification for relevant HVAC work undertaken, including, but necessarily limited to: - All required federal, state and local certification, and - Certification by NATE (or an accepted industry equivalent): - For New construction: at least 25% of the company’s start-up technicians - For Existing buildings: at least 25% of the company’s technicians |  |
| --- | --- |
| Primary Elements Listing of federal, state, and local certifications required. If organization equivalent to NATE is used, provide documentation of ACCA’s recognition of equivalency. Location of records. Identify who organizes and maintains the records. |  |
| Sample Policy #1 | There are no other federal, state, or local certifications required for Bob’s employees. NATE certificates for the Start-Up technicians are located in their employee file located in the Office Manager’s office. The Operation’s Manager ensures that at least 25% of the Start-Up and Service technicians are NATE Certified. The VP of Operations is tasked with ensuring that 25% of our technicians are NATE certified. |
| Sample Policy #2 | Bob’s Heating & Air received notification from ACCA that the XXXX certification is deemed equivalent to NATE. This record is maintained in the company’s QA Program Requirements file located in the President’s office. |

| QA-4.1 INTERACTIONS WITH BUILDING OWNERS AND HOMEOWNERS [ACCA 5 QI – 2010] The contractor shall: - Identify potential defects or deficiencies in the HVAC system or the building, - Establish options to resolve any defects or deficiencies, - Confer with the building owner or builder over work to be performed, including equipment/material to be installed by the contractor. - Ensure that work performed materially meets the written contractual agreement between the customer and the company, and - Address customer complaints. |  |
| --- | --- |
| Primary Elements Written documentation that addresses all five QA sub-elements. Location of records. Identify who organizes and maintains the records. |  |
| Sample Policy | Bob’s Heating & Air follows very specific steps when interacting with a customer. They are as follows: - Prior to working on any existing HVAC system or installing a new one, a technician will review the plan of the existing or proposed system. For existing systems, they will go to the location of the system and conduct a visual evaluation. This will allow them to point out any obvious defects or deficiencies in the existing system, or any possible problems with the system the customer had in mind for their particular situation. The technician can then choose to perform any number of tests to gauge system performance; they can include measuring the pressure drop across the coil cable, traversing, flow grid test, temperature rise test, water pressure drop test, superheat/subcooling test, electrical testing, combustion testing, blower door test, etc. - Once a defect/deficiency has been identified, the contractor will present the customer with all possible solutions to the problem. Options will be presented in writing. - Before work begins, the contractor will speak with the building owner or builder to corroborate the scope of the work to be performed, as well as the materials and equipment that will be installed by the contractor. - As the project is nearing completion, the contractor will evaluate the work done/work remaining, to ensure that all aspects of the contractual agreement will be met. - Should the customer want to file a complaint, our company has a dedicated Complaint Specialist (CS). The CS will call or meet with the customer to document the complaint extensively. Depending on the type of complaint, the resolution may include: further work being done, some aspect of the project being redone, a filing of a customer service report in the employee file, or suspension of the employee. Once the complaint is resolved, the CS will present the customer with a sheet restating the complaint and how it was addressed. The complaint is not resolved until the customer signs off. |

| Sample Policy | Disclosure Statement: All HVAC installations should be installed to meet the ACCA 5 QI Standard’s minimum requirements. All HVAC systems and equipment are presented to the client in that way. If the client declines to have the system installed to the minimum requirements, then they must acknowledge that they choose to accept an installation that is non-standard_________ (client initials) and further acknowledge they understand the risks associated with a non-standard installation_________ (client initials). All records of interactions with each customer are located in their customer file. Technicians and Installers are responsible for transmitting their onsite measurements, observations, and other information electronically to the office. This information is reviewed by the Service Manager, supplemental actions (ordering parts, customer follow-up, etc.) are executed, and upon completion of all necessary actions, the file is closed. Files are maintained on the company’s local area network, printed copies of files are available on request. |
| --- | --- |

| QA-4.2 WARRANTIES [ACCA 5 QI – 2010] Contractors offering warranties shall provide written documentation to the customer which includes at a minimum provisions addressing: - Scope of the warranty, including coverage and exclusions, - Whether the warranty is transferable, - The effective date of the warranty and any expiration dates, and - Contact information for resolution of warranty claims. |  |  |
| --- | --- | --- |
| Primary Elements Written documentation that addresses all four QA sub-elements: - Equipment manufacturer’s warranty - Contractor’s warranty on work Location of records. Identify who organizes and maintains the records. |  |  |
| Sample Policy | Bob’s Heating & Air provides all customers with a copy of their equipment warranties and warranties on work performed. The Dispatcher is responsible for keeping the originals in the records room in each installation/project file. Each set of documents with warranty information has the following sections highlighted for the customer: - Warranty scope (equipment, labor, parts, and exclusions) - Transferability of warranty - Effective date and expiration date - Contact information for warranty: Bob’s Heating and Air 123 Main Street Comfort City, XXXX 00001 Phone: 800-555-1212 Fax: 888-555-1212 Email: theboss@Bob’shvac.com |  |

| QA-4.3 SERVICE AGREEMENTS [ACCA 5 QI – 2010] Contractors offering service agreements shall provide customers written documentation which include at a minimum provisions addressing: - Scope of the service agreement, including coverage and exclusions, - Whether the service agreement is transferable by either party, - The effective date and expiration date of the service agreement, and - Contractor contact information. |  |
| --- | --- |
| Primary Elements Written documentation that addresses all four QA sub-elements: - Equipment maintenance requirements - Maintenance services offered Location of records. Identify who organizes and maintains the records. |  |
| Sample Policy | Bob’s Heating & Air’s service agreements clearly state the terms of the agreement (what is covered and what is not), the duration of the agreement (date it starts and date it ends), that the agreement is non-transferable (by either party), and the following contact information is listed: Bob’s Heating and Air 123 Main Street Comfort City, XXXX 00001 Phone: 800-555-1212 Fax: 888-555-1212 Email: theboss@Bob’shvac.com The Dispatcher is responsible for keeping the signed copies of the service agreement in the records room in each customer’s file. |

- Appendix A:  Bob’s Heating and Air Compiled Sample Policy

The Steering Committee for the ACCA Quality Assured Program offers this sample policy for your use.  This appendix lifts sample policies from the previous sections and compiles them here in one place.  This is offered as a way to easily cut-and-paste a single document.

It is expected that this sample policy will be extensively edited to more accurately reflect how your company operates on a routine basis.  Thus, you should feel free to add to these policies in order to modify and mold them until they describe “how we do it here.”  The policies must still reflect the primary elements of each of the QA Elements, but how you do it is up to you.

Bob’s Heating and Air: Quality Assured Policy Manual

Business Prerequisites

Licensing or Registration Policy (QA Element 1.1)

Bob’s Heating & Air located at 123 Main Street, in The City is required by the state of XXXX to have a State Mechanical License and a State Business License.  Additionally, YYYY County requires the company to have a County Business License.

Our licenses are located on the wall, behind the reception desk, at the entrance of our office suite.

The VP of Operations is responsible for renewing the State Mechanical License every three years. It is the duty of the President of Bob’s Heating & Air to renew the state business registration annually.

Insurance Policy (QA Element 1.2)

Bob’s Heating & Air is required to have general liability, automotive, and worker’s compensation insurance policies by the state of XXXX.  Performance bonds are purchased when are required for projects in excess of $1,500,000 or if stated in a particular project contract.

Insurance policies are filed in the ‘Insurance’ folder located in cabinet 1A in the files room in the back of the office suite.  A copy of each contract that has bonding requirements is filed in the ‘Bonding Required’ folder located in cabinet 1A, in addition to being in the regular Project File kept in the same files room.

It is the duty of the President of Bob’s Heating & Air to renew the policies annually.

Code Requirements Policy (QA Element 1.3)

Bob’s Heating & Air complies with the codes cited by the authority having jurisdiction (AHJ), XXXX (state) and YYYY County.  The cited codes are the 2012 IRC and the 2012 UMC, including the county amendments.

For work performed in other states or AHJs, Bob’s will coordinate with that AHJ to ensure copies of the applicable code are on hand, understood, and available.

Copies of the code are kept in our reference library in the staff conference room.

Compliance achieved by:

- Educating team leaders & conducting inspections of random samples of their jobs (see QA-2.4 Quality Installation)

- Maintaining a close working relationship with the building department

- Using examples of ‘red tag’ or other noted instances of failure to comply in weekly team leader meetings

- The Operations Officer coordinates annually with local code official (per instance for our-of-area prospects) to learn of any new or unique code changes / amendments

Refrigeration Certifications, Training and Equipment Policy (QA Element 1.4)

Bob’s Heating & Air provides all recover/recycle and evacuation equipment to technicians working with refrigerants.  In order to work with refrigerants, a technician must possess the applicable 608 card.  The Service Manager ensures a copy of the card is maintained cards in the employee’s company file, maintained in the Dispatcher’s office.

All new technicians must demonstrate to the Service Manager the ability to:

- Use of recover/recycle and evacuation equipment,

- Store and transport refrigerants,

- Use a micron gauge (technician supplied), and

- Change the vacuum pump oil (disposal in compliance with QA-1.5 Hazardous Waste Policy).

- Records of refrigerant handling:

  - Type (R-22, R-410A, etc.)

  - Volume, by weight

  - Date

  - Location (and zone if applicable)

Hazardous Materials Regulations Policy (QA Element 1.5)

Bob’s Heating & Air commonly works with hazardous materials (HazMats) such as lubricants, mercury, PCBs, and refrigerants.  In order to safely work with these materials, Bob’s distributes a binder of Material Safety Data Sheets (MSDS) for all hazardous materials with which we work.  We obtain the MSDS via the following methods:

- Requesting them directly from the manufacturer at the time of the purchase

- Online MSDS databases such as www.msdsxchange.com

MSDS have the following information clearly addressed on them: substance/material identification and manufacturer business info; composition and ingredients; hazards identification; first aid measures; firefighting measures; accidental release measures (spills); handling and storage; exposure controls/personal protection; physical and chemical properties; stability and reactivity; toxicological information; ecological information; disposal considerations; transport information; regulatory information; other information.  Though not all MSDSs contain all of this information, the majority of MSDS for the materials we work with do, to a significant degree.  Our Chief Safety Officer reviews the MSDS and provides supplemental guidance for when information is missing from the MSDS.

MSDS are kept in large, clearly labeled binders on each work vehicle.  This allows easy access to essential information for all employees that will need to work with hazardous materials.  They are organized by function (lubricant, refrigerant, PCB, mercury…), then manufacturer, then product identification.

Our policies on storage, transportation, use, and disposal of hazardous materials, as well as treating and cleaning up discharges and accidental spills are based on the applicable MSDS sections.  Spills require the room to be cleared of non-essential employees until the spill is taken care of as per MSDS, manufacturer’s instructions, and/or regulatory requirements.

Transportation of hazardous materials also follows the MSDS recommended method as well as U.S. DOT requirements.

An internal safety training program led by the Chief Safety Officer is conducted in conjunction with the continuing education program on a quarterly basis, with a 1.5 hour module at the time of hiring.  This safety training is intended to (1) keep all technicians current on the proper safety procedures when handling hazardous materials that they are likely to encounter, (2) to ensure that they can readily understand the information contained on MSDS, and (3) are using the appropriate safety equipment when it is necessary.

Our disposal procedures for hazardous materials are the following:

- Capacitors are disposed of at the county hazardous materials yard

  - Install Teams and Technicians place capacitors with PCBs in a green bucket by the Dispatcher’s window.

  - The Warehouse Supervisor takes them to the county hazardous materials yard.

  - Or,

  - The Install Team Leader keeps them in a marked bin on the truck and then disposes of them when they dump trash from the job site at the junk yard next door.

  - Mercury (thermostats) are disposed of at a local wholesale distributor

  - Install Teams and Technicians place thermostats with mercury in an orange bucket by the loading bay door.

  - The Installation Manager takes them to the county hazardous materials yard.

  - Or,

  - The Install Team Leader keeps them in a marked bin on the truck and then disposes of them when they pick up materials for the next job site.

- Vacuum pump oil is disposed of at the local “Oil Change” shop.

  - Install Teams and Technicians drain their vacuum pumps into an old vacuum pump oil container painted blue.  Every morning the Helper empties it into the black oil bucket by the Warehouse Supervisor’s window.

  - The Warehouse Supervisor takes the bucket to the Oil Change shop every other Thursday or when the bucket is full.

  - Or,

  - The Install Team Leader empties the blue oil container at the Oil Change shop every Friday.

SAMPLE 1: Recovered Refrigerants

- Install Teams and Technicians place recovered refrigerant in DOT approved containers provided by Bob’s Heating and Air.

- On the tag attached to the container, the Install Team Chief or Technician must document each instance that refrigerant is recovered, noting the location and volume of refrigerant recovered.

- They take the refrigerant cylinders to the local HVAC distributor’s  where they are exchanged for empty containers (they must remove the tag from the full cylinder),

- The Install Team Chief or Technician must turn in the tag in order to receive a new cylinder of virgin refrigerant.

SAMPLE 2:

- Install Teams and Technicians place recovered refrigerant in DOT approved containers provided by Bob’s Heating and Air.

- On the refrigerant log attached to the container, the Install Team Chief or Technician must document each instance that refrigerant is recovered, noting the location and volume of refrigerant recovered.

- The Parts Runner will collect will collect the full cylinder and the refrigerant log from the Team Chief or Technician when they deliver cylinders of new refrigerant.

- The Parts Runner will provide the refrigerant log and full cylinder to the Warehouse Supervisor.

- The Warehouse Supervisor will recover the refrigerant into the 500lb shop cylinder.

- The Warehouse Supervisor will call “Refrigerant for Cash” recycling service to pick up the full cylinder and drop off an empty.

- The Warehouse Supervisor will maintain the logs for all refrigerants recovered and issued.

Personal protective equipment required by Bob’s Heating and Air is listed in the Safety Programs Policy.

Business Operations

Employment Policy (QA Element 2.1)

Bob’s Heating & Air complies with federal employment eligibility requirements by completing and retaining Form I-9 and a copy of the employee’s identification in their employee file located in the Office Manager’s office.

Bob’s complies with XXXX state law and conducts a criminal background and reference check before offering employment to an applicant.

During reference checks, the Operations Manager calls the reference to:

- The relationship listed with the applicant is correct,

- Confirm the applicants dates of employment, if applicable, and

- Request any information that may be useful when considering the employment of the applicant.

As applicable, employees are assigned to duties that ensure compliance with Megan’s Law.

Safety Program Policy (QA Element 2.2)

Bob’s Heating & Air requires that every employee take a safety course during the first week of employment.

- Safety in the workplace for office employees,

- Safety in the field for all technicians and installers, and

- Behind the wheel safety for all employees with drivers licenses

This comprehensive course is taught by the HR Vice President and lasts for 2 hours.  Following that, every employee is required to complete 4 hours of safety training per year.  All safety training records are kept in the individual employee record folder.  A master list of training provided is maintained by the HR Vice President, the master list provides:

- The safety training subject,

- The instructor’s name,

- The company employees who attended.

Bob’s requires the following personal protection equipment (PPE):

- Safety glasses

- Safety harness, in circumstances required by OSHA

- Plastic/latex gloves (sealants)

- Hearing protection

- Hard hats, in circumstances required by OSHA

- Leather gloves (sheet metal)

- Steel toed shoes

Bob’s provides the safety glasses, safety harness, plastic gloves, and hearing protection.

Employees are required to provide their hard hat, leather gloves, and safety shoes.  These items are expected to be in good condition or to be replaced by the employee at the discretion of Field Supervisor.

The list of PPE required by each employee is reviewed at the time of hiring.  A signed copy of this record is kept in the Safety Program’s records (HR Vice President’s office).

Fleet Management Policy (QA Element 2.3)

Bob’s Heating & Air complies with XXXX state requirements for our fleet to undergo an annual emissions test for each vehicle.  Additionally, all company vehicles are registered with XXXX Department of Transportation, and all employees who drive these vehicles are required to have a Class C license.  Our company employs Ace Automotive for fleet maintenance and keeps copies of all maintenance records in our records room (in addition to the records kept by Ace).  The records are kept current by our Operations Officer and are kept in the filing cabinet designated Fleet Documentation.

As per XXXX state requirements, all company vehicles are marked as required in Federal Regulation 390.21 (company name and US DOT number).

Quality Installation Policy (QA Element 2.4)

Bob’s Heating & Air ensures that every system installation is compliant with ACCA 5 QI.  System designers and installation technicians are thoroughly trained and comfortable with the requirements of the Standard.

HVAC System Design

The system designer opens an installation file that includes the load calculation, expanded manufacturers equipment performance data, duct sketches, duct sizing calculations, and the AHRI certificate for the system to be installed.  The file is then given to the Operations Manager.

- Ventilation requirements are calculated using ASHRAE 62.2

- Heating and cooling loads are calculated using ACCA Manual J

- Equipment is selected using the procedures in ACCA Manual S and the information from the OEM expanded performance data

- Ground heat exchangers are designed in consideration of the elements in ACCA 5 QI §3.4.1.ii

- An AHRI certificate is printed and filed for all systems

HVAC System Installation

HVAC system installation measurements are recorded on the company start-up checklist (for ENERGY STAR homes, their checklist is used) in accordance with the procedures defined in the Technician’s Guide for Quality Installations.  Homes intended for ENERGY STAR qualification have well marked pre-drilled holes where static pressure measurements may be taken.  For HVAC system replacements, the lead technician is responsible for demonstrating how to operate the thermostat, how to change the filter, and for leaving the Bob’s Heating and Air Customer Information Sheet that explains the warranty, maintenance program, and the company contact information. Upon completion of a system installation, the Team Chief submits the company startup checklists (and other supporting documentation as needed) to the Operations Manager.

Ensuring HVAC System Installation Compliance

The Operations Manager visits every job upon completion to ensure compliance with this policy.  Projects lasting more than three (3) days receive at least one jobsite visit during the visit.  As a minimum, projects lasting longer than five (5) days receive an onsite job review at least on every third day.

OR….

The Operations Manager visits every job upon completion to ensure compliance with this policy.

OR….

The Operations Manager randomly selects 30% of all jobs to visit (upon completion) to ensure compliance with this policy.

The Operations Manager follows the verification process found in the ACCA Standard 9 Quality Installation Verification Protocols (ACCA 9 QIvp) to measure compliance.   The Operations Manager observes the protocols for the ‘Administrator/Verifier’ in ACCA 9 QIvp.  Their duties are:

- Inspect every installation file for completeness and compliance with QI Standard

- Random in-field verification of system installations.

The Operations Manager follows the evaluation protocols of Section 5.0 of the QIvp and uses the tables/checklist in that section for assistance in determining compliance.

Upon verification completion the Operation’s Manager is responsible for storing the installation file in the customer’s file located in the Dispatcher’s office.

Quality Maintenance Policy (QA Element 2.5)

Bob’s Heating & Air only performs residential work, so we follow ACCA 4 QM for maintenance of HVAC systems. Bob’s uses a series of checklist that can be followed by our technicians in the field to conduct comprehensive system maintenance.  The technician conducts a thorough inspection of the HVAC equipment per the checklists in the standard.  Upon completion of the inspection, the technician will inform the owner/building operator of any deficiencies (code violations as well as the performance objectives for the equipment based on make, model, start-up date, etc.).  The technician will then perform the corrective actions approved by the customer (the customer is the person paying for the approved work).

Bob’s maintenance checklists document the items cited in §5.1.1 to §5.1.7 of ACCA 4 QM (the home’s equipment inventory, equipment inspection checklist, code violations, performance objectives, relevant external conditions, and inaccessible items).  Corrective actions are based on the procedures from OEM.

Every employee that services and maintains HVAC equipment is given a copy of ACCA 4 QM.

QM documentation is kept in the service record of each client, and the service records are maintained by the Dispatcher in the Maintenance/Service file cabinet in the Dispatcher’s office.

Quality Restoration Policy (QA Element 2.6)

Bob’s Heating & Air follows strict steps when restoring and HVAC system that are in accordance with ACCA Standard 6.  They are the following:

- A trained technician will conduct an initial survey to assess the practicality of a restoration, and to see determine possible problems.  The survey includes:

  - Visual system cleanliness evaluation (interior and exterior),

  - Inspection of drains, and

  - Inspection of room where the system is housed.

- System performance shall be measured in order to compare the pre- and post-restoration performance.  If there is a possible impact to the customer’s health, an indoor air quality expert will be asked to provide a formal report.

- Bob’s Heating & Air follows the protocols for system restoration that are found in Section 5.0 (System Restoration Protocol) of the ACCA 6 QR which covers the following topics:

  - Worker protection,

  - General principles (containment, physical dislodgment, particle transport, contaminant removal),

  - Cleaning equipment (particulate collectors, pressurized air supply, HEPA filtered canister vacuums, duct surface brushing systems, power washers),

  - Cleaning processes and procedures (continuous pressure differential,

  - Particulate collectors,

  - Fibrous glass duct insulation cleaning method,

  - Uniformity in cleaning,

  - Fine particulate extraction,

  - Electric heating and electric reheat coils,

  - Removal of contaminated materials from HVAC systems),

  - Building isolation,

  - Work sequence,

  - Protecting cleaned surfaces,

  - Disposal of existing HVAC filters,

  - Cleaning of HVAC system components and subsystems (cooling/heating coils, fan blades and fan housing, motors, air handling units and interior heat exchanger cabinet surfaces, drain pans and drain line, HVAC airflow control components, registers, diffusers, and grilles, ductwork systems, rigid ducts, fiberglass lined or duct board ductwork, flexible non-metallic ducts), and

  - duct service openings and closures (metal ducts, service opening and closure fiberglass duct board ducts, flexible ducts).

- The Team Leader verifies the cleanliness of the system by turning it on, measuring performance, recording measurements, and comparing it to pre-cleaning performance.  If an expert was called in during the initial system evaluation, an independent third party reviewer will be asked to review the system for health reasons.

- The Operations Manager visits every job upon completion to ensure compliance with this policy.  Projects lasting more than three (3) days receive at least one jobsite visit during the visit.  As a minimum, projects lasting longer than five (5) days receive an onsite job review at least on every third day.

OR….

The Operations Manager visits every job upon completion to ensure compliance with this policy.

OR….

The Operations Manager randomly selects 30% of all jobs to visit (upon completion) to ensure compliance with this policy.

- Upon completion, the owner (or their agent) visually inspects the system cleanliness.

- Bob’s Heating & Air follows the exact directions for post cleaning products and usage that are found in Section 7.0 (Post Cleaning Products and Usage) of Standard 6 which covers the following topics: resurfacing products, closure products (mastics, tapes for rigid fiberglass duct board ducts and other openings), sanitizers, disinfectants and other antimicrobial products.

Every employee that will be doing restoration work has a personal copy of Standard 6 issued to them.

Restoration records are kept in the service record of each client, and the service records are maintained by the Dispatcher in the Maintenance/Service file cabinet in the Dispatcher’s office.

Instrument/Measurement Tools Policy (QA Element 2.7)

Bob’s Heating & Air provides each technician with a digital anemometer, a flow grid, a digital temperature gauge, and a multimeter (voltmeter and ammeter) in order to properly install, maintain, restore, and test HVACR systems in the field.  Digital meters are calibrated according to the manufacturer’s recommendation.  We have one blower door that is kept in the office and can be signed out by technicians prior to using it.

The Operations Manager keeps the calibration and service records for company owned instruments and equipment in the calibration file located in his/her office.  Each Technician is in charge of keeping their instruments calibrated (if needed) and serviced per equipment manufacturer’s guidelines.  The Operations Manager also spot-checks the technicians’ personal tools to ensure that they are properly calibrated and serviced.

Training and Certification

Continuing Education Policy (QA Element 3.1)

Bob’s Heating & Air provides a comprehensive continuing education program delivered by the Continuing Education Officer.  Our salespeople and designers take a three hour course every six (6) months.  Our technicians are required to take a three (3) hour technical education course every quarter.  A small comprehension assessment is given by the instructor at the end of each course.

In order for a technician to opt out of the continuing education course, they must maintain their current NATE certification.  They must provide us with their NATE number (to be included in the personnel file) for corroboration, or we verify each technician’s Continuing Education Units annually.

If a technician misses a quarterly training class they must make up the continuing education hours by attending a training class provided by an OEM, an HVAC distributor, a tech school, or ACCA and submitting corroboration of attendance.  These training classes cannot replace more than three (3) hours of our company’s in-house continuing education courses to fulfill the six (6) and twelve (12) hour annual requirements.

Records of the courses (class material, subjects covered, date, instructor, attendees) are kept in the records room of our office suite in the continuing education cabinet.  These records shall be maintained for at least five (5) years.  The calendar of future courses is kept in the staff break room.

Certification Policy (QA Element 3.2)

There are no other federal, state, or local certifications required for Bob’s employees.

NATE certificates for the Start-Up technicians are located in their employee file located in the Office Manager’s office.  The Operation’s Manager ensures that at least 25% of the Start-Up and Service technicians are NATE Certified.

The VP of Operations is tasked with ensuring that 25% of our technicians are NATE certified.

Customer Relations

Customer Relations Policy (QA Element 4.1)

Bob’s Heating & Air follows very specific steps when interacting with a customer.  They are as follows:

- Prior to working on any existing HVAC system or installing a new one, a technician will review the plan of the existing or proposed system.  For existing systems, they will go to the location of the system and conduct a visual evaluation.  This will allow them to point out any obvious defects or deficiencies in the existing system, or any possible problems with the system the customer had in mind for their particular situation.  The technician can then choose to perform any number of tests to gauge system performance; they can include measuring the pressure drop across the coil cable, traversing, flow grid test, temperature rise test, water pressure drop test, superheat/subcooling test, electrical testing, combustion testing, blower door test, etc.

- Once a defect/deficiency has been identified, the contractor will present the customer with all possible solutions to the problem.  Options will be presented in writing.

- Before work begins, the contractor will speak with the building owner or builder to corroborate the scope of the work to be performed, as well as the materials and equipment that will be installed by the contractor.

- As the project is nearing completion, the contractor will evaluate the work done/work remaining, to ensure that all aspects of the contractual agreement will be met.

- Should the customer want to file a complaint, our company has a dedicated Complaint Specialist (CS).  The CS will call or meet with the customer to document the complaint extensively.  Depending on the type of complaint, the resolution may include: further work being done, some aspect of the project being redone, a filing of a customer service report in the employee file, or suspension of the employee.  Once the complaint is resolved, the CS will present the customer with a sheet restating the complaint and how it was addressed.  The complaint is not resolved until the customer signs off.

Disclosure Policy:  All HVAC installations should be to ACCA 5 QI Standard, all HVAC systems and equipment are presented to the client in that way. If the client declines to have the system installed to the minimum tolerances per metric, then they must acknowledge that they choose to accept an installation that is non-standard_________ (client initials) and further acknowledge they understand the risks associated with this type of proper installation_________ (client initials).

All records of interactions with each customer are located in their customer file.  Technicians and Installers are responsible for transmitting their onsite measurements, observations, and other information electronically to the office.  This information is reviewed by the Service Manager, supplemental actions (ordering parts, customer follow-up, etc.) are executed, and upon completion of all necessary actions, the file is closed.  Files are maintained on the company’s local area network, printed copies of files are available on request.

Warranty Policy (QA Element 4.2)

Bob’s Heating & Air provides all customers with a copy of their equipment warranties and warranties on work performed.  The Dispatcher is responsible for keeping the originals in the records room in each installation/project file.  Each set of documents with warranty information has the following sections highlighted for the customer:

- Warranty scope (equipment, labor, parts, and exclusions)

- Transferability of warranty

- Effective date and expiration date

- Contact information for warranty:

Bob’s Heating and Air

123 Main Street

Comfort City, XXXX 00001

Phone: 	800-555-1212

Fax: 	888-555-1212

Email: 	theboss@Bob’shvac.com

Service Agreements Policy (QA Element 4.3)

Bob’s Heating & Air’s service agreements clearly state the terms of the agreement (what is covered and what is not), the duration of the agreement (date it starts and date it ends), that the agreement is non-transferable (by either party), and the following contact information is listed:

Bob’s Heating and Air

123 Main Street

Comfort City, XXXX 00001

Phone: 	800-555-1212

Fax: 		888-555-1212

Email: 	theboss@Bob’shvac.com

The Dispatcher is responsible for keeping the signed copies of the service agreement in the records room in each customer’s file.