Department                                               Date                     Reviewer:

|  | Expected Practice | Yes/No |  | Comments |
| --- | --- | --- | --- | --- |
| 1. | Dedicated Phlebotomy station - Privacy is maintained. |  |  |  |
|  | - Clean surface or counter available for supplies. |  |  |  |
|  | - Alcohol-based hand antiseptic available. |  |  |  |
|  | - Hand wash sink within short distance for use. |  |  |  |
|  | - Chairs for patient and phlebotomist have cleanable surface. |  |  |  |
|  | - Sharps placed in locked cabinet at night. |  |  |  |
|  | - Sharps disposal container close by point of use. |  |  |  |
|  | - Containers sealed for disposal |  |  |  |
|  | - Discarded when ¾ full |  |  |  |
|  | - PPE close by point of use. |  |  |  |
|  | - Supplies stored appropriately. |  |  |  |
|  | - Separate clean supplies from sterile supplies-both separated from soiled supplies or other contaminants |  |  |  |
|  | - Stored in plastic bins/no shipping containers used for storage |  |  |  |
|  | - If centrifuge present, it has lid. Locate centrifuge in dedicated lab area, no food allowed. |  |  |  |
|  | - No personal decorations/items/toys – reduce clutter in station. |  |  |  |
|  | - Must have a designated “clean” area for food items (e.g. patient or employee). |  |  |  |
| 2. | Phlebotomist aware of current Billings Clinic policy and procedures about phlebotomy and their contact (phlebotomy lead or supervisor for questions). |  |  |  |
| 3. | Phlebotomist Practices - Wears lab jacket and clean gloves to perform blood draw. |  |  |  |
|  | - Lab jacket is to be buttoned up and sleeves down |  |  |  |
|  | - Lab jacket to be changed when soiled or every 7 days |  |  |  |
|  | - Introduces self to patient and asks for patient identifier. |  |  |  |
|  | - Patient name & birthdate |  |  |  |
|  | - Performs hand hygiene. |  |  |  |
|  | - Disposable tourniquet used. (Tourniquets should not remain on patient any longer than 2 minutes) |  |  |  |
|  | - Clean gloves worn. |  |  |  |
|  | - No reuse of gloves |  |  |  |
|  | - Discard gloves after use |  |  |  |
|  | - Appropriate blood draw device used with safety attachment. |  |  |  |
|  | - Disinfect venipuncture site. |  |  |  |
|  | - 70% alcohol swab, use circular motion, allow to air dry |  |  |  |
|  | - Do NOT touch cleaned site with bare finger. If site is touched, repeat the disinfection prior to needle entry. |  |  |  |
|  | - Perform venipuncture. |  |  |  |
|  | - Collect samples in correct order |  |  |  |
|  | - Release tourniquet before withdrawing needle. Discard tourniquet. |  |  |  |
|  | - Hold gauze in place. Don’t bend arm as it can cause bruise. |  |  |  |
|  | - Discard used needle in sharps disposal container. |  |  |  |
|  | - Label specimen and complete forms in presence of patient. |  |  |  |
|  | - Check insertion site for bleeding. |  |  |  |
|  | - Ask patient how they are feeling. |  |  |  |
|  | - Thank patient and tell them the procedure is completed. |  |  |  |
|  | - Clean station after every patient. |  |  |  |
|  | - Use approved EPA-registered disinfectant (e.g. Sanicloth) to clean phlebotomy chair, counter or surface where supplies were located, anything touched by patient or phlebotomist |  |  |  |
|  | - Remove/discard gloves and perform hand hygiene. |  |  |  |

References:

- Billings Clinic P/P #1 Performing a Phlebotomy Procedure. Billings Clinic Laboratory Phlebotomy Procedure Manual.

- World Health Organization (WHO)  2010 Guidelines on drawing blood: Best Practices in Phlebotomy