Asset Management Plan for Yorkeys Knob SES


	Summary

The main building is a high set ex school building, built around 1960’s. At the rear of the main building is a colourbond metal tray clad shed which serves as car and boat storage. 

The building is well suited to Yorkeys Knob as the area can be inundated and high set building has the best chance of being above the flood. The SES facility is located in a Council park, has plenty of space, but lacks toilets and showers. The occupants use the toilets in the adjoining building.

Vandalism is a problem in this park, although a resident caretaker is located in an adjacent Council house. 
It is reported that fires have been lit at the rear of the SES; so far with little damage.


	
	[image: image3.emf]Cumulative whole of life expenditure

$-

$50,000

$100,000

$150,000

$200,000

$250,000

$300,000

$350,000

$400,000

$450,000

1 4 7 10 13 16 19 22 25 28 31 34

Year


[image: image4.emf]Building name Yorkeys Knob SES

Type Disaster Response

Address  58-54 Wattle Street - Community Centre Park

Suburb Yorkeys Knob

Asset No 51897 (Shed - 52905)

Parent Asset 69523

Land Title Lot 2 RP737261

Parcel No 86019

Land type (Freehold / In Trust) Council Freehold Land

Characteristic Useful life 50

Construction (timber, block, Hi-set, Low-set) Timber High Set

Construction date (estimate if unknown) 1960's

Internal area (m2) 9.3 x23 m2 214 m2



	[image: image5.emf]Replacement cost 280,000 $  

WDV 77,959 $    

Frequency of use (score = 1-10) 6



	
	[image: image1.jpg]



Main Building – Street Frontage

	[image: image2.jpg]



Main Training Area

Comment 

This building setup is well suited to its function, well appointed, and, in good condition. The AC roof is being  replaced with colourbond Custom Orb, and the outside is to be repainted by Council. A life of at least another 20 years is easily achievable.

Recommendations

The facility requires vandal proofing, own toilets and showers and completion of building in and vermin proofing, and some fairly minor improvements.

Reference Documents

Inspection & estimates #1238854
	
	Concerns

· The building is subjected to vandalism.

· Requires air conditioning to the main training area for the hot humid summer.

· Disabled (wheel chair) access upstairs is not possible and would be hard to implement.

· The downstairs area is not vermin proof, and children in the adjoining play group throw in sand from the sandpit through the block vents.

Improvements
· Install screens on street side windows for vandal proofing. 

· Complete the build in of the downstairs section to vermin proof and seal the building.

· Install showers and toilets in the downstairs section.

· Install steel beams to remove 2 stumps downstairs.

· Add another bay onto the garage to fit the boat; or two bays to fit the SES trailer as well, in the shed instead of under the building.

· Partition the main training area for personal lockers and PPE storage; include internal painting.

· Air condition the main training area.



	
	
	

	
	
	











#1238872 v2
Page 1 of 1

