Name:____________________________

# Medical Record #:__________________

Date:_________________________

# Correct Use of MDI and Spacer with Mask

Below are the steps for the correct use of a metered dose inhaler (MDI) and spacer with MASK. Caregiver/patient should perform the following:

1.  Shake the canister for 5 seconds.

2.  Prime MDI. (Varies depending on MDI brand, see package insert.) In general:

-If MDI not used in 2 weeks or has been dropped: spray 2 puffs into air

-If MDI never used before spray 3 puffs into air

3.  Insert the MDI into the spacer.

4.  Place the mask on the face, covering the mouth and nose completely.

5.  Look for a seal around the mouth and nose and the mask.

6.  Press down the top of the canister to release 1 puff of medicine.

7.  Allow the child to take 6 breaths with the mask in place.

8.  Wait 1 minute after 6th breath before giving another puff of the medicine.

9.   Repeat steps 4 through 8 depending on how many puffs are indicated on the prescription.

# Cleaning Instructions

- Remove mask and the rubber end of spacer where the MDI fits.

- Rotate spacer mouthpiece counter-clockwise and lift up to remove.

- Lift the valve off the clear posts at the end of the chamber.

- Soak the parts in warm water with clear, liquid detergent for about 15 minutes.

- Rinse in clean water and shake to remove excess water.

- Allow all parts to air dry. DO NOT dry with a towel.

- To reassemble, hold chamber upright and place valve over clear posts. Replace spacer mouthpiece and turn it clockwise until it locks into place.

- Replace the back rubber end onto the spacer.