| SOP Name: Using Sanidate 5.0 |  |  |  | Created On: August 11, 2021 |  |  |  | Last Modified: August 11, 2021 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Purpose: Ensure Equipment and Tools Used for Harvesting are Clean |  |  |  |  |  |  |  |  |
| Who performs the task? | Where is the task performed? |  |  |  |  | When is the task carried out? |  |  |
| Equipment/ Tools / Supplies: Measuring utensil, clean water, Sanidate 5.0, brush, soap, test strips (optional) |  |  | Record / Form Used: Cleaning Log |  |  |  | Created By: CAFF |  |
|  |  |  |  |  |  |  | Modified By: CAFF |  |
| Health & Safety Warnings: Use gloves & goggles Read & adhere to label warnings Review MSDS/SDS sheets for all warnings |  | Cautions: Ensure general working area is clean before beginning cleaning |  |  | Terminology: MSDS/SDS – available in MSDS/SDS binder in storage cabinet |  |  |  |
| Procedure: - Dress in PPE: safety glasses, rubber gloves and protective clothing - Spray tools and equipment with hose on a strong setting to remove and loosen dirt - Wash dirt from tools and equipment with brush and soap - Rinse tools and equipment with water - Prepare Sanidate 5.0 sanitizer solution: - Per 1 gallon container, use 2 teaspoons to 2 tablespoons of Sanidate 5.0 and clean water (147– 500 parts per million (ppm) of peroxyacetic acid). - Follow instructions on test strips and test sanitizer solution for appropriate ppm concentration. (optional) - Use the prepared solution to apply Sanidate 5.0 to all surfaces thoroughly. Treated surfaces must remain visible wet for one minute. - Allow all items and surfaces to air dry. Do not rinse. - Record date and initial Cleaning Log. ___________________________________________________________________________________ For reference: From Sanidate 5.0 sublabel B label page 12: 1. Remove all food prior to sanitization of food storage areas. 2. Prior to use of this product, remove visible contamination with a cleaner or other suitable detergent and rinse with potable water. 3. Apply 1.6 to 5.4 fl. oz. of SaniDate 5.0 to 5 gallons of potable water (147– 500 ppm of peroxyacetic acid) with a mop, cloth, sponge, or hand trigger spray so as to wet all surfaces thoroughly. 4. Treated surfaces must remain visibly wet for one (1) minute. 5. Allow items and/or surfaces to air dry. Do not rinse. |  |  |  |  |  |  |  |  |